High Input Impedance Voltage-mode Universal Filter and Its modification as Quadrature Oscillator Using VDDDAs

Sunti Tuntrakool1, Montree Kumngern1, Roman Sotner2, Norbert Herencsar3, Peerawut Suwanjan4, Winai Jaikla4

1Department of Telecommunications Engineering, Faculty of Engineering,

King Mongkut’s Institute of Technology Ladkrabang, Bangkok 10520, Thailand,

2Department of Radio Electronics, Faculty of Electrical Engineering and Communication,

Brno University of Technology (BUT), Technicka 3082/12, Brno, 616 00, Czech Republic,

3Department of Telecommunications, Faculty of Electrical Engineering and Communication,

Brno University of Technology (BUT), Technicka 3082/12, Brno, 616 00, Czech Republic,

4Department of Engineering Education, Faculty of Industrial Education,

King Mongkut’s Institute of Technology Ladkrabang, Bangkok, 10520, Thailand,

*E-mail: sunti.tu@kmitl.ac.th, kkmontre@kmitl.ac.th, sotner@feec.vutbr.cz, herencsn@feec.vutbr.cz, winai.ja@kmitl.ac.th

Received ; revised ; accepted

In this paper, the second order universal voltage-mode filter using voltage differencing differential difference amplifiers (VDDDAs) is proposed. It has high input impedance voltage-mode biquad filter with orthogonal tune of natural frequency and quality factor. The proposed filter simultaneously provides five filter responses: low-pass (LP), high-pass (HP), band-reject (BR), all-pass (AP) and band-pass (BP) in the same circuit topology. The natural frequency and quality factor can be tuned electronically and orthogonally DC bias current. The output impedance at output nodes HP, AP and BR has low impedance which can connect to other circuit without the use of voltage buffers. The proposed filter consists of three VDDDAs, one grounded resistor and two grounded capacitors. This makes the proposed filter suitable for integrated circuit development. With slightly modifying the proposed filter, the voltage-mode qudrature sinusoidal oscillator with low output impedance and independent control of condition of oscillation (CO) and frequency of oscillation (FO) is achieved. The results shown in this paper are from PSPICE simulation and experiment to validate the proposed circuits.

Keywords: Analog filter, VDDDA, Voltage-mode, Single input-Multiple output, Oscillator

# 1 Introduction

The oscillator circuit and analog active filter are also popular and standard topic for circuit design. They are an important requirement for application in electrical and electronic system and also very popular in using for circuit design of continuous-time analog signal processing. There are many fields that using filters circuit such as communications, measurement, and instrumentation, and control systems1. Especially, researchers have been very considerably interest in several functions filter which is called universal filter or multifunction filter. The single-input, multiple-output (SIMO) is the most popular analog filter where different output filter functions can be simultaneously realized by the same circuit topology2.

Using of active building block for circuit design is very popular in use. It gives the flexibility for designer to realize the high performance circuit using minimum number of active element3-7. With mentioned features, the principle of active building blocks for both current and voltage mode circuit are introduced by Biolek et al.6. VDDDA8 is one of the interests. It allows interesting utilization and design of more profitable or more exacting application especially the electronic controllability. From literature reviews, it is found that not much research using VDDDA have been published for instance the voltage-mode first order all pass filter8, oscillator8-10. The excellent multiple-input multiple-output (MIMO) voltage-mode universal filter using VDDDA was proposed in10-14. These filters can provide complete standard transfer functions with high inputs and low outputs impedance. The natural frequency and quality factor can be electronically and orthogonally controlled. However, these filters cannot be considered as universal filter because all five filter responses cannot be simultaneously provided. The multiple-input single-output (MISO) voltage mode filter was introduced in15. This filter consists of single active building block, single MOS transistor and two floating capacitors. It can provide five filter responses depending on the appropriate selecting input voltage. The natural frequency and quality factor can be electronically tuned. However, this filter cannot be easy to cascade without the use of voltage buffers. Moreover, the use of floating capacitors is not attractive for integration.

This contribution presents a SIMO voltage-mode filter with high input impedance, emphasizing on use of VDDDAs. The proposed filter composes of three active elements, one grounded resistor and two grounded capacitors which are suitable for fabricating in monolithic chip or off-the-shelf implementation. The proposed filter can provide five standard functions such as low-pass, high-pass, band-reject, all-pass and band-pass. The quality factor and natural frequency can be electronically and orthogonally adjusted. With slight modification of the proposed filter, the voltage-mode quadrature oscillator with low output impedance is achieved.

# 2 Principle of Operation

## 2.1 VDDDA Overview

The principle of VDDDA was introduced by Biolek et al.6. Later, Herencsar et al. proposed the internal construction of VDDDA using CMOS technology8. Symbol and equivalent circuit of VDDDA are shown in Fig. 1 (a) and (b) respectively, where V+ and V- are the voltage input terminals which will be converted to be the current at z terminal by transconductance (gm). It is generally tuned by bias current and the differential voltage at terminal z, n and p will be send to w terminal with the unity voltage gain. For ideal VDDDA, it has low output impedance at w terminal and high input impedance at terminals V+, V-, z, n and p. The characteristics matrix equation of ideal VDDDA is described in (1) below.

…(1)

Fig. 1 VDDDA (a) Symbol (b) Equivalent circuit

## 2.2 High Input Impedance Voltage-mode Filter Using VDDDAs

Fig.2 is the proposed second order filter consisted of three VDDDAs, one grounded resistor and two grounded capacitors. The proposed filter provides simultaneously five filter responses: HP, LP, BR, AP and BP (BP1 and BP2) with high input impedance. Moreover, the output nodes for HP, AP and BR responses exhibit low output impedance. Considering an ideal VDDDA, routine analysis of the proposed filter provides the following voltage transfer functions

,             	                          …(2)

,           	                          …(3)

,    	                          …(4)

,      	             …(5)

,                                        …(6)

,		           …(7)

where

.                  	…(8)

It is found from (2)-(8) that the proposed filter is the unit gain filter, then, for any practical use, additional voltage amplifiers are needed to achieve gain from the active filter. However, the gain is probably obtained for BP1 only if specific Q is set.  The natural frequency (ω0) and quality factor (Q) of each filter response can be expressed in (9).

and            	…(9)

From (9), it is found that the quality factor can be electronically tuned via gm3 without affecting the natural frequency. Moreover, if gm1 equal to gm2, the natural frequency can be electronically adjusted without affecting the quality factor. However, it is found that the proposed circuit needs component matching conditions (i.e. gm3=1/R) for realizing AP response.

Fig. 2 Presented voltage-mode filter

The relative sensitivities of the proposed filter can be found in (10).

…,(10)MERGEFORMAT

It is found that the active and passive sensitivities are equal or less than unity in magnitude.

# 3 Non-ideal Case

Practically, the performances of the proposed filter are affected by the influences of voltage tracking error from the unity-value gain of internal differential voltage buffer and parasitic terminal impedances of VDDDA8. In this section, these parameters will be taken into account. For non-ideal case the voltage at w terminal is re-written as shown below.

…(11)

From (10), βz, βn, and βp-are the voltage error gains from z, n, p terminals to w terminal respectively. The influences of parasitic impedances of V-1, p1, n2 and n3 terminals will be negligible because of their connection to low-impedance outputs (w3 terminal) and input voltage source. The most importance parasitic impedances are the impedance at z1 (Rz1//Cz1), n1 (Rn1//Cn1), V+2 (RV+2//CV+2), z2 (Rz2//Cz2) and p3 (Rp3//Cp3) terminals. However, the parasitic impedances at z3 terminal will be negligible but the operation frequency fop should be more lower than 1/{Cz3(Rz3+R)}. The voltage transfer functions for the circuit of Fig. 3 are given in (12) – (17).

…(12)

…(13)

…(14)

…(15)

…(16)

….(17)

where

…(18)

= C1+Cz1+Cv+2+Cv+3,  = C2+Cz2+Cn1+Cp3, = Gz1+Gv+2+Gv+3 and  = Gz2+Gn1+Gp3. Also non-ideal values of ω0 and Q are found in equations (19) and (20) respectively.

…(19)

…(20)

# 4 Simulations Results

PSPICE simulations of the proposed filter in Figure 2 were performed. The implementation of the CMOS VDDDA was same in Ref.8. Parameters of a 0.18µm TSMC CMOS technology (level 7)16 with ±0.9 V voltage supply and VB = -0.35 V was used for simulation of PMOS and NMOS transistors. From Table 1, aspect ratios of PMOS and NMOS transistor are listed. It is seen that the parasitic resistances at terminals V+, V-, n, and p (Rv+, Rv-, Rn and Rp) exhibit high because they are gate resistance. Other simulated parasitic element values for each terminal (IB = 50mA) are as follows: Cv+ = 55.5 fF, Cv- = 53.2 fF, Rz = 570.54 kW, Cz = 15.4 fF, Cn = 4.24 fF and Cp = 4.25 fF. The simulated voltage error gains, βz, βn, and βp are equal to 0.997. The filter was designed with the parameters of its components as follows: C1 = C2 = 47 pF, R = 3.3 kW, IB1 = IB2 = IB3 = 50µA. It yields the natural frequency of 1.047 MHz and quality factor of 1. The theoretical pole frequency is about 1.058 MHz. From the results, the gains responses for LP, BP1, BP2 and HP of the proposed filter obtained from Fig. 2 are shown in Fig. 3. Fig. 4 and 5 are the gain response and phase response of BR and AP responses respectively. It is obviously that the proposed filter can simultaneously provide low-pass, high-pass, band-pass, band-reject and all-pass functions without modifying circuit topology. The gain response of BP2 difference IB3 is shown in Fig. 6, where IB3 was set to 20 mA, 50 mA and 200 mA. The quality factor evaluated based on the simulation results was 1.49, 1, and 0.65, respectively. This confirmed equation (9) that the quality factor can be electronically tuned by IB3 without affecting the natural frequency. High Q value can be achieved by setting IB3 as low as possible. The highest simulated Q is 26.66 (IB3 = 1 mA) and the lowest simulated Q is 0.384 (IB3 = 400 mA). Fig. 7 shows the dependence of the THD of LP filter on input voltage level. The THD is not over 1 % when the input signal is lower 650 mVp-p. In this test, sinusoidal signal with 100 kHz in-band frequency was fed into the proposed filter

| Table 1 — Dimensions of the transistors |  |  |
| --- | --- | --- |
| Transistor | W (µm) | L (µm) |
| M1-M2, NMOS (AGC) | 9 | 1.08 |
| M3-M4 | 3.96 | 1.08 |
| M5-M7 | 3.6 | 1.8 |
| M8-M11 | 0.72 | 1.08 |
| M12-M14 | 2.16 | 1.08 |

Fig. 3 Frequency responses of proposed filter LP, BP1,            BP2 and HP

Fig. 4 Gain and phase response of BR

Fig. 5 Gain and phase response of AP

Fig. 6 BP2 responses for difference IB3

Fig. 7 Dependence of output harmonic distortion of LP filter      on the input voltage

| 5 Comparison with Existing SIMO Voltage-Mode Filters Table 2 — Comparison of various SIMO voltage-mode filters |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref. | ABB | No. of ABB | No. of R+C | Grounded elements only | High input impedance | Electronic tune | Orthogonal tune of Q and w0 | Five filter responses | Low output impedances | Technology |
| [17] | DVCC | 3 | 3+2 | yes | yes | no | no | yes | no | CMOS |
| [18] | DVCC | 3 | 3+2 | no | yes | no | yes | yes | no | CMOS |
| [19] | DVCC | 2 | 3+2 | no | no | no | yes | yes | no | CMOS |
| [20] | FDCCII (Fig. 3) | 1 | 2+2 | yes | yes | no | no | no | no | CMOS |
| [21] | DDCC | 2 | 3+2 | no | no | no | no | yes | no | CMOS |
| [22] | DDCC | 3 | 2+2 | no | no | no | no | yes | no | CMOS |
| [23] | DDCC & OTA | 2 | 1+2 | yes | yes | yes | no | no | no | CMOS |
| [24] | OTA | 8 | 0+2 | yes | yes | yes | yes | no | no | CMOS |
| [25] | CFOA | 1 | 3+2 | no | no | no | yes | no | LP | commercial IC |
| [26] | DDCCTA | 1 | 1+2 | yes | yes | yes | no | no | no | CMOS |
| [27] | DDCCTA | 2 | 2+2 | yes | yes | yes | no | yes | no | CMOS |
| [28] | OTA | 8 | 0+2 | yes | yes | yes | yes | no | no | CMOS |
| [29] | DDCCTA | 3 | 0+2 | yes | yes | yes | no | yes | AP | CMOS |
| [30] | CCCCTA | 1 | 1+3 | no | no | yes | yes | no | no | BJT |
| [31] | DDCCTA | 1 | 2+2 | no | no | yes | no | no | no | CMOS |
| [32] | DDCCTA | 2 | 2+2 | yes | yes | yes | no | yes | AP | CMOS |
| [33] | FDCCII | 1 | 3+2 | no | no | no | no | yes | no | CMOS |
| [34] | DVCC | 4 | 5+2 | yes | yes | no | yes | no | no | CMOS |
| [35] | ICCII | 2 | 4+2 | no | no | no | no | no | no | CMOS |
| [36] | DVCC | 2 | 2+3 | no | no | no | no | no | no | CMOS |
| [37] | DDCCTA | 2 | 2+2 | no | yes | yes | yes | yes | no | CMOS |
| [38] | VD-DIBA | 2 | 0+2 | yes | yes | yes | no | no | HP | commercial IC |
| [39] | VDCC (Fig. 3) | 1 | 2+2 | yes | no | yes | yes | no | no | CMOS |
| [40] | VDCC | 1 | 2+2 | no | no | yes | yes | no | no | BJT |
| [41] | CCII | 2 | 3+2 | no | yes | no | no | no | no | CMOS |
| [42] | DDCCTA | 2 | 3+2 | yes | yes | yes | yes | yes | AP | CMOS |
| [43] | VDTA (Fig. 5) | 1 | 0+2 | yes | yes | yes | no | no | no | CMOS |
| [44] | CCII | 4 | 5+2 | no | yes | no | yes | yes | no | commercial IC |
| This Work | VDDDA | 3 | 1+2 | yes | yes | yes | yes | yes | HP, AP, BR | CMOS & commercial IC |

The proposed SIMO voltage-mode filter in Fig. 2 is compared with several SIMO voltage-mode filters from17-44. It is found from Tab. 2 that there are VM MISO structures having even some low-output impedance outputs25,29,32,38,42. However, their other drawbacks are in missing possibility for electronic control, requirements for floating passive elements and higher number of active elements (4 or 5). All these problems are solved in solution presented in this paper.

6 Modification of Proposed Filter As Quadrature Oscillator

By connecting node VBP1 to Vin and interconnecting terminal V+ and V- of VDDDA1 and VDDDA2 of the circuit in Fig. 2 according to the principle in Ref.45 as illustrated in Fig. 8, the voltage-mode quadrature oscillator with low output impedance can be achieved. The characteristic equation of the oscillator in Fig. 8 is obtained as

.    	           …(21)

According to (21), the frequency of oscillator (FO) and condition of oscillation (CO) are written as

, and 	                        …(22)

It is found from (22) that the FO and CO are independently and electronically controlled. The relationship of VO2 and VO1 is follows:

.                      		           …(23)

At oscillation frequency (w0), the magnitude of VO2/VO1 is written as

…(24)

It is found from (24) that the changing of gm1 or gm2 for controlling the FO causes change of amplitude VO2 and VO1 during tuning process. This phenomenon will increase the THD if amplitude reaches high levels due to the limits of dynamical range of VDDDA. However, this can be alleviated by simultaneously changing gm1 and gm2 (IB1 = IB2). As stated above, the amplitude of quadrature output voltage VO1 and VO2 is equal for all frequency. However, to unify unbalance of produced amplitudes VO1 and VO2 as well as to reduce the THD, the simple AGC circuit for amplitude stabilization can be easily applied to terminal z of VDDDA3.

- Fig. 8 Voltage-mode quadrature oscillator with

- low output impedance

The proposed oscillator in Fig. 8 was simulated with the parameters of its components as follows: C1 = C2 = 47 pF, R = 3.3 kW, Rp = 330 kW, IB1 = IB2 = 50 µA and IB3 = 51.5 µA. The W/L of NMOS in AGC is 9 s /1.08 mm. It yields the FO of 1.014 MHz. The theoretical FO is about 1.058 MHz. The results of this simulation are respectively shown in Fig. 9-10. The total harmonic distortion for VO1 and VO2 are 0.68% and 76%, respectively. Tuning of simulated and theoretical FO is shown in Fig. 11, where IB1 and IB2 are equal and were adjusted from 10 A – 300 A. The range of FO controlled from 0.31 MHz - 2.42 MHz was obtained. It is found that there is some deviation between theoretical and simulated value due to the parasitic element as analyzed in (19). The tuning of gm by adjusting IB will change the value of parasitic elements.

- Fig. 9 Quadrature output waveform

- Fig. 10 Output spectrum

Fig. 11 Tuning of FO by adjusting IB1 and IB2

7 Experimental Results

The performances of the proposed filter and oscillator were also experimentally investigated. The VDDDA was constructed from the available commercial ICs, AD830 and LM13700 as illustrated in Fig. 12. The transconductance of LM13700 is gm = IB / 2VT where VT is thermal voltage (VT @ 26 mV at room temperature). The proposed filter was firstly tested with following conditions; the supply voltage ±5 V, C1 = C2 = 5.6 nF, IB1 = IB2 = IB3 = 115 mA (gm1 = gm2 = gm3 = 2.211 mA/V) and R = 0.45 kW. With these conditions, the natural frequency and quality factor are 62.853 kHz and 1, respectively. The experimental gain response of BP2, BP1, LP, HP, BR and AP is shown in Fig. 13. The experimental natural frequency is about 61 kHz which was about 2.948 % deviated from theoretical value. The tuning of Q without affecting natural frequency is confirmed by the experimental result of BP2 filter in Fig. 14 where the value of IB3 was changed to 57.5 mA, 115 mA and 230 mA. The measurements of output voltage VBP2 is also shown in Fig. 15 where the 50 mVp-p sinusoidal voltage with 63 kHz of frequency was applied as input signal.

- Fig. 12 Internal construction of VDDDA constructed

- from an available commercial ICs

- Fig. 13 Experimental gain responses of the proposed filter

- Fig. 14 Experimental gain response of BP2

- for different value of IB3

The proposed oscillator in Fig. 8 was tested with following conditions; the supply voltage 5 V, C1 = C2 = 5.6 nF, IB1 = IB2 = IB3 = 115 A (gm1 = gm2 = gm3 = 2.211 mA/V) and R=0.54 k. With these conditions, the FO is 62.9 kHz. Figure 16 shows the measured output voltage where the experimental FO was about 61.86 kHz which was about 1.579 % deviated from theoretical value. It is also found that the output voltages Vo1 and Vo2 are quadrature sinusoidal signal.

- Fig. 15 Measurement of VBP2 at frequency 63 kHz

Fig. 16 Measurement of output voltage and its spectrum

where IB1 = IB2 = 115 A

8 Conclusions

In this contribution, voltage-mode bi-quad filter has been proposed. The advantages of the proposed filter are as follows. Firstly, it can perform variety of filters i.e. low-pass, high-pass, band-pass, band-reject and all-pass functions. Secondly, the quality factor and the natural frequency can be electronically and orthogonally controlled. Finally, the filter has high input impedance. Moreover, the output voltage terminals for functions high-pass, band-reject and all-pass are low output impedance. The proposed filter consists of three VDDDAs, one grounded resistor and two grounded capacitors, which are attractive for either IC implementation. With slightly modifying the proposed filter, the voltage-mode quadrature oscillator low output impedances is achieved. The CO and FO can be independently and electronically tuned. Moreover, the ratio of amplitudes VO1 and VO2 is constant on the tuning of FO if IB1 and IB2 are simultaneously tuned. Simulation results confirmed theoretical anticipation and validity of the synthesis. The power consumption for proposed filter is 0.343 mW and for proposed oscillator (with AGC circuit) is 0.346 mW. Moreover, the experimental results using available commercial ICs (AD830 and LM13700) are included and they meet very well with theoretical presumptions.

# Acknowledgements

The authors would like to thank the anonymous reviewers for providing valuable comments which helped improve the paper substantially. This work was supported by the Higher Education Research Promotion and National Research University Project of Thailand, Office of the Higher Education Commission, Grant No. A118-59-028 and Czech Science Foundation projects under No. 14-24186P.

# References

- Sedra A S & Smith K C, Microelectronic circuits, 6th edition, (Oxford University Press, USA), 2011.

- Psychalinos C, Analog Integr Circuits Signal Process, 67 (2011) 201.

- Jantakun A & Jaikla W, Indian J Pure Appl & Phys, 53(8) (2015) 557.

- Chaichana A, Jantakun A, Kumngern M & Jaikla W, Indian J Pure Appl & Phys, 53(7) (2015) 470.

- Yuce E & Minaei S, Int. J. Circuit Theory & Appl, 42(7) (2014) 659.

- Biolek D, Senani R, Biolkova V & Kolka Z, Radioengineering, 17(4) (2008) 15.

- Kubánek D, Khateb F, Tsirimokou G & Psychalinos C, Circuits Syst Signal Process, 35 (2016) 2003.

- Herencsar N, Šotner R, Metin B, Koton J; & Vrba K. VDDDA - New 'voltage differencing' device for analog signal processing, International Conference on Electrical and Electronics Engineering, Bursa, Turkey, (2013).

- Chaichana A, Jaikla W, Suwanjan, P & Tuntrakool S, A new quadrature sinusoidal oscillator for telecommunication system using VDDDAs, International Conference on Intelligent Informatics and Biomedical Sciences (ICIIBMS), (2015).

- Tuntrakool S, Kumngern M & Jaikla W, VDDDAs-based voltage-mode multiphase sinusoidal oscillator, International Conference on Industrial Application Engineering, (2016).

- Koton J, Herencsar N, Vrba K & Metin B, The VDDDA in multifunction filter with mutually independent Q and ω0 control feature, International Conference on Electrical and Electronics Engineering (ELECO), (2013).

- Koton J, Herencsar N, Vrba K & Metin B, Analog Integr Circuits Signal Process, 81(1) (2014) 53.

- Sangyaem S, Siripongdee S, Jaikla W & Khateb, F, Optik, 128(1) (2016) 14.

- Siripongdee S & Jaikla W, Single VDDDA-based voltag-mode multifunction second order filter for analog signal processing, International Conference on Intelligent Informatics and Biomedical Sciences (ICIIBMS), (2015).

- Herencsar N, Cicekoglu O, Šotner R, Koton J & Vrba K, Analog Integr Circuits Signal Process, 76(2) (2013) 251.

- http://lgjohn.ecen.ceat.okstate.edu/5263/processparam/t4bk_lo_epi-params.html

- Minaei S & Yuce E, Circuits Syst Signal Process, 29 (2010) 295.

- Chiu W Y, Horng J W, Lee H & Huang C C, IEEE International Symposium on Electronic Design, Test and Applications, Ho Chi Minh City, Vietnam, (2010).

- Horng J W, Analog Integr Circuits Signal Process, 62 (2010) 407.

- Chiu W Y & Horng J W, Indian J Eng & Mat Sci, 18 (2011) 97.

- Kac¸ar F & Yes¸i, A, Analog Integr Circuits Signal Process, 63 (2010) 137.

- Chiu W Y, Horng J W, Guo Y S & Tseng C Y, DDCCs based voltage-mode one input five outputs biquadratic filter with high input impedance, International Symposium on Integrated Circuits, Singapore, (2011).

- Udorn N, Duangmalai D, Noppakarn A, High input impedance current controlled voltage-mode universal filter using DDCC and OTA, IEEE International Conference on Vehicular Electronics and Safety (ICVES), (2011).

- Kumngern M, Suwanjan P & Dejhan K., Electronically tunable voltage-mode SIMO OTA-C universal biquad filter, Asia-Pacific Conference on Communications (APCC2011), (2011).

- Horng J W, Hou C L, Huang W S & Yang D Y,, circuits and systems, 2 (2011) 60.

- Tangsrirat W & Channumsin O, Radioengineering 20 (2011) 905.

- Channumsin O, Pukkalanun T & Tangsrirat W, Microelectron J, 43(8) (2012) 555.

- Kumngern M, Suwanjan P & Dejhan K, Int J Electron, 100 (2013) 1118.

- Tangsrirat W, Channumsim O & Pukkalanun T, Microelectron J, 44 (2013) 210.

- Singh, S V, Maheshwari S, Tomar R S & Chauhan D S, Single input four output voltage-mode biquad filter with electronic tuning, International Conference on Multimedia Signal Processing and Communication Technologies, (2013).

- Channumsin O & Tangsrirat W, Microelectron J, 44 (2013) 1084.

- Chen H P, IET Circuits, Devices and Systems, 8 (2014) 280.

- Mohan J, Chaturvedi B & Maheshwari, S. Advances in Electrical Engineering, (2014) Article ID 514019

- Ismail M U & Arif B, Single input multi output digitally reconfigurable biquadratic analog filter, International Conference on Systems Informatics, Modelling and Simulation, (2014).

- Chen, H.-P., Voltage-mode multifunction biquadratic filter with one input and six outputs using two ICCIIs, The Scientific World Journal, 2014, Article ID 432570

- Chen, H.-P., Chiu, Y.-L., Chung, C.-K., Chou, C.-C., Voltage-mode multifunction filter with single input and three outputs based on single plus-type DVCC, International Conference on Information Science, Electronics and Electrical Engineering, 2014, p. 1838-1841.

- Chen H P, Wang S F, Huang W Y & Hsieh M Y, IEICE Electronics Express, 11 (2014) 20140234.

- Jaikla, W., Biolek, D., Siripongdee, S., Bajer, J., High input impedance voltage-mode biquad filter using VD-DIBAs. Radioengineering, 2014, vol. 14, no. 03, p. 0914-0921.

- Fırat, K., Abdullah, Y., Koray., G., Design and experiment of VDCC-based voltage mode universal filter, Indian J Pure Appl & Phys, 2015, vol. 53, p. 341-349.

- Sagbas M, Ayten U E, Koksal M & Herencsar N, Electronically tunable universal biquad using a single active component, International Conference on Telecommunications and Signal Processing (TSP), (2015).

- Yuce, F., Yuce, E., J Circuits Syst, Comput, 24(4) (2015), 1550047.

- Chen H P & Wang S P, AEU-Int J Electron Commun, 70 (2016) 491.

- Alaybeyog˘lu, E & Kuntman, H, Analog Integr Circ Sig Process,89 (2016) 675.

- Horng J W & Chiu W Y, Indian J Pure Appl & Phys, 54 (2016) 557.

- Bajer J, Vavra J, Biolek D & Hajek K, Low-distortion current-mode quadrature oscillator for low-voltage low-power applications with non-linear non- inertial automatic gain control. In Proceedings of the ECCTD 2011. Linkoping, Sweden, (2011).